# Jazz and Blues — Home Recording Cheat Sheet

Quick-reference guide  |  Sheet 08

# Reliable Starting Setup

• Record uncompressed WAV at 24-bit, usually 44.1 or 48 kHz; match the project and delivery requirements.

• Set input gain so typical peaks stay comfortably below 0 dBFS (often around −12 to −6 dBFS); never clip.

• Start with a pop filter and a microphone about 6–10 inches from a solo singer, adjusting for tone, room, and delivery.

• Use closed-back headphones when overdubbing. Choose a low buffer for monitoring if your system can handle it; increase buffer for mixing.

• Reduce reflections and noise with placement, soft furnishings, and mic direction before relying on plugins.

• Save a clean, dry recording even if monitoring with reverb, compression, or tuning.

• Label tracks and takes, save often, and keep at least one separate backup.

# Genre Focus

Jazz: prioritize natural dynamics and room tone; use minimal mic setups when possible; check bleed and phase.

# Session Checklist

☐ Room and noise checked

☐ Sample rate and bit depth confirmed

☐ Headphone mix comfortable

☐ No clipping during loudest passage

☐ Reference and guide tracks ready

☐ Takes named and saved

☐ Dry files exported and backed up

# Troubleshooting

## Distortion

Lower preamp gain and repeat the take; clipped audio cannot reliably be restored.

## Echo or boxy tone

Move away from walls and corners; use absorption near reflection points.

## Latency

Use direct monitoring or a smaller buffer and disable high-latency plugins while tracking.

## Plosives or sibilance

Move slightly off-axis, use a pop filter, and adjust distance before processing.